ATA Photonics Engine

Technical Summary — v0.4

Meryem Atayeva / ATA Photonics

## What

ATA Photonics Engine is a simulation and benchmarking platform for photonic AI accelerator architectures, starting with Mach-Zehnder Interferometer (MZI) based neural networks.

It provides a clean Python framework for training, decomposing, and benchmarking photonic neural networks against equivalent electronic baselines, with a hardware realism layer grounded in published silicon photonics process data.

## Why photonics

- Matrix-vector multiplications in the optical domain at the speed of light

- Theoretical energy per MAC: ~0.01 pJ photonic vs ~1.0 pJ electronic (x100 advantage)

- Propagation latency through MZI mesh: sub-nanosecond

- No von Neumann bottleneck: weights are encoded as physical phase angles on-chip

## Architecture

Each hidden neuron is a programmable MZI:

input x  ->  phase shifter (phi = W*x + b)  ->  MZI interference (I = cos2(phi/2))  ->  photodetector

Weights are encoded as voltages on thermo-optic heaters deposited over silicon waveguides. The Clements decomposition converts any trained weight matrix into a sequence of (theta, phi) angles that directly program the physical chip.

## Hardware realism model

| Parameter | Value | Unit | Notes |
| --- | --- | --- | --- |
| Insertion loss | 0.5 | dB/MZI | 220 nm SOI |
| Thermal tuning | 20 | mW/pi | Thermo-optic |
| Detector noise sigma | 0.01 | normalised | Shot + thermal |
| Bandwidth | 10 | GHz | Thermo-optic limit |
| MZI footprint | 500 | um2 | 50 x 10 um |
| Wavelength | 1550 | nm | Telecom C-band |
| Energy/MAC (photonic) | ~0.01 | pJ | Theoretical |
| Energy/MAC (electronic) | ~1.0 | pJ | 8-bit digital |

## v0.4 benchmark

Three datasets: XOR, Two-Moons, MNIST 0v1 (PCA-16). Matched architecture for fair comparison.

Key result: Photonic model achieves comparable accuracy to an electronic baseline while showing a 100x theoretical energy advantage per inference operation.

## Roadmap

| Version | Milestone |
| --- | --- |
| v0.4 | Benchmark engine, 3 datasets, hardware realism model |
| v0.5 | Multi-layer photonic networks, coherent detection |
| v0.6 | Backpropagation through Clements decomposition |
| v0.7 | Tape-out candidate: GDS layout + foundry DRC |
| v1.0 | First fabricated chip: measurement vs simulation |

## Key references

- Shen et al., Deep learning with coherent nanophotonic circuits, Nature Photonics 2017

- Clements et al., An optimal design for universal multiport interferometers, Optica 2016

- Hamerly et al., Experimental investigation of MZI network performance, Advanced Photonics 2019

ATA Photonics — simulation to silicon